Knowledge Transfer Document: Speed Monitoring System

Overview

This project implements a Speed Monitoring System using a Raspberry Pi 4B. It integrates:

- TSR20 Nanoradar for real-time speed detection

- CP Plus IP Camera (RTSP stream) to capture overspeeding events

- 64x48 RGB P10 LED display for live visual feedback

- A Flask-based web application with REST API to set speed limits and view violations - A virtual environment for dependency isolation and a systemd service to ensure the system starts on boot

Hardware Used

- Raspberry Pi 4B

- TSR20 Nanoradar (RS485 to USB via pyserial)

- CP Plus IP Camera (RTSP stream)

- 6x RGB P10 Panels (32x16, arranged 2 columns × 3 rows)

- 5V power supply for P10 LED panels

- Wiring connectors and optional relays

P10 Display Pin Configuration

HUB75 signal mapping to Raspberry Pi GPIO:

R1 -> GPIO 17

G1 -> GPIO 18

B1 -> GPIO 22

R2 -> GPIO 23

G2 -> GPIO 24

B2 -> GPIO 25

A -> GPIO 7

B -> GPIO 8

C -> GPIO 9

OE -> GPIO 2

CLK -> GPIO 3

LAT -> GPIO 4

GND -> GND

TSR20 Nanoradar Serial Port Detection

To detect the serial port the radar is connected to:

$ ls /dev/ttyUSB*

Software Environment Setup

1. Create Virtual Environment:

$ python3 -m venv myenv

$ source myenv/bin/activate

$ pip install opencv-python flask numpy pyserial waitress pillow

2. Clone and Build RGB LED Matrix Library:

$ git clone https://github.com/hzeller/rpi-rgb-led-matrix.git

$ cd rpi-rgb-led-matrix

Knowledge Transfer Document: Speed Monitoring System

$ make build-python PYTHON=$(which python3)

$ sudo make install-python

Flask Web Interface

The Flask web interface supports:

1. Speed Limit Input (via UI and API)

2. Viewing Screenshots of overspeeding events

3. REST API access to automate or remote control the system 4. Company branding via static/logo.png

Speed limit is stored in settings.txt as a single number (e.g., 60)

REST API Endpoints

| Method | Endpoint | Description | |--------|---------------------|--------------------------------------------| | GET | /get_speed_limit | Returns the current speed limit | | POST | /set_speed_limit | Sets a new speed limit ({ "limit": 60 }) | | GET | /violations | Lists saved screenshots | | GET | /violation/<name> | Returns a specific screenshot image |

Screenshot Logic

If the detected speed exceeds the speed limit:

- A frame is captured from the RTSP camera stream using OpenCV - Saved in a folder named datetime.now().strftime("%Y%m%d") - Filenames: HHMMSS.jpg

- Example: /home/pi/20250621/153412.jpg

Autostart on Boot (systemd Service)

1. Create service file:

$ sudo nano /etc/systemd/system/speedmonitor.service

Add:

[Unit]

Description=Speed Monitoring System

After=network.target

[Service]

ExecStart=/home/pi/myenv/bin/python /home/pi/speedmonitor/final_code.py WorkingDirectory=/home/pi/speedmonitor/

Restart=always

User=pi

Environment="PATH=/home/pi/myenv/bin"

[Install]

WantedBy=multi-user.target

2. Enable service:

$ sudo systemctl daemon-reexec

Knowledge Transfer Document: Speed Monitoring System

$ sudo systemctl enable speedmonitor

$ sudo systemctl start speedmonitor

Project Folder Structure

speedmonitor/

??? final_code.py

??? settings.txt

??? flask_web/

? ??? app.py

? ??? static/

? ? ??? logo.png

? ??? templates/

? ??? index.html

??? 20250621/

? ??? 153412.jpg

??? myenv/

??? requirements.txt

Appendix: Full Source Code (final_code.py)

import cv2

from datetime import datetime

import os

import time

import threading

import serial

import subprocess

from rgbmatrix import RGBMatrix, RGBMatrixOptions, graphics

from PIL import Image, ImageDraw, ImageFont

# ====================== Simulated Speeds ======================

ser = serial.Serial('/dev/ttyUSB0', baudrate=9600, timeout=1)

# ====================== Configurations ======================

RTSP_URL = 'rtsp://admin:admin12345@192.168.1.250:554/cam/realmonitor?channel=1&subtype=0' SETTINGS_FILE = "/home/pi/flask-webpage/settings.txt"

# ====================== RGB Matrix Setup ======================

options = RGBMatrixOptions()

options.rows = 16

options.cols = 32

options.chain_length = 6

options.parallel = 1

options.hardware_mapping = 'classic-pi1'

options.pwm_bits = 11

options.brightness = 80

options.show_refresh_rate = 0

options.scan_mode = 1

Knowledge Transfer Document: Speed Monitoring System

options.gpio_slowdown = 3

options.disable_hardware_pulsing = True

matrix = RGBMatrix(options=options)

canvas = matrix.CreateFrameCanvas()

# ====================== Fonts & Colors ======================

font = graphics.Font()

font.LoadFont("/home/pi/rpi-rgb-led-matrix/fonts/9x15.bdf")

font_small = graphics.Font()

font_small.LoadFont('/home/pi/rpi-rgb-led-matrix/fonts/6x9.bdf')

font_large = graphics.Font()

font_large.LoadFont('/home/pi/rpi-rgb-led-matrix/fonts/9x13.bdf')

color_safe = graphics.Color(0, 255, 0)

color_warn = graphics.Color(255, 165, 0)

color_alert = graphics.Color(255, 0, 0)

font_path = "/usr/share/fonts/truetype/dejavu/DejaVuSans-Bold.ttf"

font_size = 24

pil_font = ImageFont.truetype(font_path, font_size)

# ====================== Panel Mapping ======================

def get_panel_offset(panel_name):

mapping = {

'top_left': 0,

'top_right': 32,

'mid_left': 64,

'mid_right': 96,

'bot_left': 128,

'bot_right': 160

}

return mapping[panel_name]

# ====================== Draw Speed Split ======================

def draw_large_split_text(canvas, text, color, top_left_x, top_left_y, mid_left_x, mid_left_y):

image = Image.new("RGB", (32, 32), "black")

draw = ImageDraw.Draw(image)

draw.text((0, 0), text, font=pil_font, fill=(color.red, color.green, color.blue))

top_half = image.crop((0, 0, 32, 16))

bottom_half = image.crop((0, 16, 32, 32))

for y in range(top_half.size[1]):

for x in range(top_half.size[0]):

r, g, b = top_half.getpixel((x, y))

canvas.SetPixel(top_left_x + x, top_left_y + y, r, g, b)

Knowledge Transfer Document: Speed Monitoring System

for y in range(bottom_half.size[1]):

for x in range(bottom_half.size[0]):

r, g, b = bottom_half.getpixel((x, y))

canvas.SetPixel(mid_left_x + x, mid_left_y + y, r, g, b)

# ====================== Helper Functions ======================

def get_speed_limit():

try:

with open(SETTINGS_FILE, "r") as f:

return int(f.read().strip())

except Exception as e:

print(f"Settings error: {e}")

return 70

def update_display(speed, speed_limit):

canvas.Clear()

if speed <= speed_limit - 10:

color = color_safe

alert = "Safe Speed"

elif speed_limit - 10 < speed <= speed_limit:

color = color_warn

alert = "Warning"

else:

color = color_alert

alert = "SLOW DOWN!"

speed_text = str(speed)

draw_large_split_text(

canvas,

speed_text,

color,

get_panel_offset('top_left'),

0,

get_panel_offset('mid_left'),

0

)

x_kmh = get_panel_offset('mid_right') + 2

graphics.DrawText(canvas, font_small, x_kmh, 12, color, "km/h")

alert_width = graphics.DrawText(canvas, font_small, 0, 0, graphics.Color(0, 0, 0), alert)

x_alert = get_panel_offset('bot_left') + 64 - (alert_width // 2)  graphics.DrawText(canvas, font_small, x_alert, 12, color, alert)

matrix.SwapOnVSync(canvas)

def read_speed():

data = ser.read(4)

if len(data) < 4:

Knowledge Transfer Document: Speed Monitoring System

return 0

if data[0] == 0xFC and data[1] == 0xFA and data[3] == 0x00:

return data[2]

return 0

def overlay_speed_info(frame, radar_speed, speed_limit):

text = f"Speed: {radar_speed} km/h | Limit: {speed_limit} km/h"  font = cv2.FONT_HERSHEY_SIMPLEX

font_scale = 0.8

font_thickness = 2

(text_width, text_height), baseline = cv2.getTextSize(text, font, font_scale, font_thickness)

x, y = 20, 40

cv2.rectangle(frame, (x - 10, y - text_height - 10), (x + text_width + 10, y + 10), (255, 255, 255), -1)

cv2.putText(frame, text, (x, y), font, font_scale, (0, 0, 0), font_thickness)  return frame

def capture_screenshot(speed, speed_limit):

timestamp = datetime.now().strftime('%H%M%S')

date_str = datetime.now().strftime('%Y%m%d')

folder_name = f"/home/pi/{date_str}"

os.makedirs(folder_name, exist_ok=True)

filename = f"{timestamp}.jpg"

save_path = os.path.join(folder_name, filename)

try:

cap = cv2.VideoCapture(RTSP_URL)

if not cap.isOpened():

print("? Failed to connect to camera")

return

start_time = time.time()

ret = False

while time.time() - start_time < 5:

ret, frame = cap.read()

if ret:

break

time.sleep(0.1)

if not ret:

print("? Frame capture failed")

return

annotated_frame = overlay_speed_info(frame, speed, speed_limit)  cv2.imwrite(save_path, annotated_frame)

print(f"? Saved violation: {save_path}")

except Exception as e:

print(f"?? Capture error: {e}")

finally:

if 'cap' in locals():

Knowledge Transfer Document: Speed Monitoring System

cap.release()

def record_hourly_stream():

while True:

current_time = datetime.now()

date_str = current_time.strftime('%Y%m%d')

hour_str = current_time.strftime('%H')

folder_path = f"/home/pi/{date_str}/videos"

os.makedirs(folder_path, exist_ok=True)

output_filename = f"{date_str}_{hour_str}.mp4"

output_path = os.path.join(folder_path, output_filename)

ffmpeg_command = [

'ffmpeg',

'-rtsp_transport', 'tcp',

'-i', RTSP_URL,

'-t', '3600',

'-vcodec', 'copy',

'-acodec', 'copy',

output_path

]

print(f"?? Starting FFmpeg recording: {output_path}")  try:

subprocess.run(ffmpeg_command, check=True)

print(f"? Finished recording: {output_path}")  except subprocess.CalledProcessError as e:

print(f"?? FFmpeg failed: {e}")

time.sleep(5)

# ====================== MAIN ======================

if __name__ == "__main__":

try:

threading.Thread(target=record_hourly_stream, daemon=True).start()  while True:

speed = read_speed()

speed_limit = get_speed_limit()

update_display(speed, speed_limit)

if speed > speed_limit:

print(f"?? Violation: {speed} km/h")

capture_screenshot(speed, speed_limit)

else:

print(f"? Safe speed: {speed} km/h")

time.sleep(0.2)

except KeyboardInterrupt:

print("\n?? System stopped")

Knowledge Transfer Document: Speed Monitoring System

Autostart on Boot (All Components)

We?ll create three `systemd` services so that each major component launches on boot and runs independently.

1. Speed Monitoring Service (final_code.py):

$ sudo nano /etc/systemd/system/speedmonitor.service

[Unit]

Description=Speed Monitoring System (final_code.py)

After=network.target

[Service]

ExecStart=/home/pi/myenv/bin/python /home/pi/speedmonitor/final_code.py WorkingDirectory=/home/pi/speedmonitor/

Restart=always

User=pi

Environment="PATH=/home/pi/myenv/bin"

[Install]

WantedBy=multi-user.target

2. Flask Web App Service (app.py):

$ sudo nano /etc/systemd/system/flaskweb.service

[Unit]

Description=Flask Web UI for Speed Monitoring

After=network.target

[Service]

ExecStart=/home/pi/myenv/bin/python /home/pi/speedmonitor/flask_web/app.py WorkingDirectory=/home/pi/speedmonitor/flask_web/

Restart=always

User=pi

Environment="PATH=/home/pi/myenv/bin"

[Install]

WantedBy=multi-user.target

3. REST API Service (restapi.py):

$ sudo nano /etc/systemd/system/restapi.service

[Unit]

Description=REST API Service for Speed Monitoring

After=network.target

[Service]

ExecStart=/home/pi/myenv/bin/python /home/pi/speedmonitor/flask_web/restapi.py WorkingDirectory=/home/pi/speedmonitor/flask_web/

Restart=always

User=pi

Knowledge Transfer Document: Speed Monitoring System

Environment="PATH=/home/pi/myenv/bin"

[Install]

WantedBy=multi-user.target

To enable all services:

$ sudo systemctl daemon-reexec

$ sudo systemctl enable speedmonitor

$ sudo systemctl enable flaskweb

$ sudo systemctl enable restapi

$ sudo systemctl start speedmonitor

$ sudo systemctl start flaskweb

$ sudo systemctl start restapi